# NTP computer

The NTP computer is standard PC with a PPS interface card installed. The operating system is FreeBSD-3.4. A driver for the PPS interface card and a NTP packet filter has been integrated with the base OS. The NTP-4 distribution with a reference clock driver, a microstepper daemon, the SSH distribution and a set of additional utilities completes the software set.

# PPS interface hardware

The PPS interface is designed for a cesium or rubidium frequency standard with a PPS pulse. The interface card contains PPS interrupt logic, a interrupt latency counter, a microstepper and a PPS output. The card also contains a clock generator that can provide a 14 MHz clock syncronized to a 10 MHz signal from the frequency standard. The intended use for this 14 MHz clock is to remove the 14 MHz crystal on the PC motherboard replacing it with the 14 MHz output using a short cable. The main benefit from this hardware modification is to remove the temperature instability caused by the motherboard crystal.

The PPS interface card has a internal 10 MHz oscillator that replaces the external 10 MHz signal automatically when it is disconnected. The internal oscillator is only used to create the 14 MHz PC clock.

The main logic on the PPS interface card is built using a Altera FPGA circuit. The FPGA configuration is stored in a onboard Atmel serial eeprom. The eeprom FPGA configuration can be updated using a download connector onboard the PPS card.

## Specifications

| PC interface | ISA-bus |
| --- | --- |
| Interrupt | Irq 5, 7, 10 or 14 |
| I/O address | 0x3e0 – 0x3e7 |
| Inputs | PPS input. Positiv edge, min +1V |
|  | Frequency. 10 MHz, min 1V rms |
| Outputs | PPS output. Positive edge, 100 ns, min +1V |
|  | 13.968256 MHz to PC motherboard, min 1V rms |
| Onboard leds | 4 external and 2 internal leds for PPS signals etc. |

The PPS and frequency inputs have optional onboard 50 ohm terminators. With the terminator jumpers removed, the input impedanace is 1000 ohms.

## PPS interface blocks

## Using the card with a GPS receiver

A GPS receiver provides only a PPS signal. In these cases the PPS interface can be configured to use the internally generated 14 MHz signal instead of the external 10 MHz. Without a external 10 MHz signal, the 14 MHz signal is generated using a internal 10 MHz crystal. When using a crystal controlled reference frequency, the microstepper and the PPS output is to be considered unreliable. For example, consider the effect of a frequency error of 50 ppm. The error is in the worst case accumulated for a entire second resulting in a total error of 50 us.

## Updating the FPGA configuration

FPGA configuration bitstreams is downloaded to the onboard Atmel eeprom using a cable connecting a PC printerport to the onboard download connector. The cable used is connected like this :

DB25 PC printerport						Download connector

Data           1 -------------------------------------------------------------------- 1
Clock       14 -------------------------------------------------------------------- 3
GND        25 -------------------------------------------------------------------- 7
SE(GND) 24 -------------------------------------------------------------------- 10

The bitstream file is in Intelhex format and is built using Altera MaxPlus II. The Altera output file is converted to the proper format using the Atmel CPS program.

## Internal registers

The onboard registers are as follows :

| Read/write | I/O address | Function |
| --- | --- | --- |
| Read | 03e0 | Latency register, byte 0. ( lsb ). |
| Read | 03e1 | Latency register, byte 1. |
| Read | 03e2 | Latency register, byte 2. |
| Read | 03e3 | Read activates updating of the latency register. |
| Read | 03e4 | Read deactivates updating of the latency register. |
| Read | 03e5 | Read activates PPS interrupts. |
| Read | 03e6 | Read deactivates PPS interrupts. |
| Read | 03e7 | Read resets PPS interrupts. |
| Write | 03e0 | PPS output, byte 0. ( lsb ). |
| Write | 03e1 | PPS output, byte 1. |
| Write | 03e2 | PPS output, byte 2. |
| Write | 03e3 | Microstepper setting, byte 0. ( lsb ). |
| Write | 03e4 | Microstepper setting, byte1. |
| Write | 03e5 | Microstepper setting, byte 2. |
| Write | 03e6 | General output port. Bit 0 : Clock signal when configuring the frequency synthesizer. Bit 1 : Data signal when configuring the frequency synthesizer. Bit 2 : 0 for external clock signal. 1 for internal clock signal. |

## Programming the internal registers

Microstepper

The incoming PPS pulse is always routed through the microstepper. The microstepper can delay the internal PPS pulse by up to 1 second in 100 ns increments. The value written to the microstepper register is calculated as : 2 – desired delay i 100 ns increments. For example, if you want to delay the PPS pulse by 50 microseconds the programmed value should be 2 – ( 10 * 50 ) = -498.

PPS output

The PPS output counter can delay the microstepped PPS pulse by up to 1 second before feeding it to the PPS output. The PPS output is programmed exactly like the microstepper. The intended use of the PPS output is to keep it syncronized to the system clock enabling a external instrument to measure the offset between the incoming PPS pulse and the system clock.

Latency counter

The latency counter counts clock cycles from the active edge of the microstepped PPS pulse. It is read by the PPS interrupt routine and used when the interrupt latency compensation is calculated. Before the latency registers are read, updating of the register from the latency counter must be deactivated by reading address 03e4. When the registers are read, updating is activated again by reading address 03e3.

# Microd, clock monitoring daemon

The monitoring daemon performs 3 tasks :

  - Monitor offset and dispersion from the ntp server and accept or reject ntp queries depending on these values

  - Receive offset/drift compensation values from external sources and update the microstepper according to these values. Recalculation of present offset is performed at regular intervals.

  - Receive status queries and send responses to these queries.

The daemon communicates using SSL, but can also be configured to use unauthenticated clear text communication.

## Configuration and command line syntax

The daemon is configured using a configuration file specified on the command line. If no configuration file is specified /etc/microd.conf is used.

A typical configuration file :

updateinterval		600
clientport		273
ppsdevice		/dev/ppsd0
offsetfile		/etc/microd.offset
ntpfilter		yes
allowresponses	yes
allowcontrol		no
deviceaddress		127.127.32.0
maxoffset		0.000050
maxdispersion		0.00100
usessl			yes

certfile			/etc/certificate/Microd_cert.pem
keyfile			/etc/certificate/Microd_key.pem
cafile			/etc/certificate/Cacert.pem
cipher			RC4-MD5

updateinterval

Specifies the interval, in seconds, between recalculations of the present offset using supplied offset and drift values.

clientport

Specifies the tcp port the daemon listens on.

ppsdevice

The device file used for communication with the PPS kernel driver.

offsetfile

The file where received offset and drift values are stored.

ntpfilter

Yes/True activates monitoring of the ntp server and received ntp queries. Ntp queries are accepted/rejected depending on the present ntp server offset and dispersion.
No/False decativates the ntp server monitoring.

allowresponses

Only used when ntpfilter=yes.

Yes/true accepts incoming ntp response packets.
No/False rejects incoming ntp response packets.

allowcontrol

Only used when ntpfilter=yes.

Yes/True accepts incoming ntp control packets.
No/False rejects incoming ntp control packets.

deviceaddress

Only used when ntpfilter=yes.

Specifies the ntp peer address for the PPS clock driver.

maxoffset

Only used when ntpfilter=yes.

Specifies the ntp offset limit where ntp queries are rejected.

maxdispersion

Only used when ntpfilter=yes.

Specifies the ntp dispersion limit where ntp queries are rejected.

usessl

Yes/true specifies that all communication is to be performed using SSL.
No/False specifies that all communication is to be performed in clear text.

certfile

Only used when usessl=yes.

Specifies the file containing the daemon certificate.

keyfile

Only used when usessl=yes.

Specifies the file containing the daemon private key.

cafile

Only used when usessl=yes.

Specifies the file containing the CA-certificate.

cipher

Only used when usessl=yes.

Specifies the type of crypto used.

## Microd communication protocol

External clients connects to microd using a SSL or a cleartext tcp connection. The tcp port used is specified in the configuraton file.

The interface details are described in the header file microd.h.

The message structure is described as C-code structures below.

/*

* Clockevent from pps device driver.

*/

struct ppsclockev {

struct timeval	tv;

u_int			serial;

};

/*

* Statistic summary from pps device driver.

*/

struct ppsstat {

int		minhw;

int		maxhw;

int		minsys;

int		maxsys;

};

/*

* Microstepper offset/drift data block.

*/

struct ClockData {

int		Offset;		/* Signed offset in ns    */

int		Drift;		/* Signed drift in ps/24h */

short		Leap;			/* Leap second. -1, 0, +1 */

short		Status;		/* Other status bits      */

};

/*

* Microstepper data block with offset/drift/freq drift.

*/

struct ClockData {

int		Offset;		/* Signed offset in ns    */

int		Drift;		/* Signed drift in ps/24h */

int		FreqDrift;		/* Signed fdrift in fs/s/24h */

short		Leap;			/* Leap second. -1, 0, +1 */

short		Status;		/* Other status bits      */

};

/*

* Union of all commands.

*/

union Cmds {

short			Ecode;	/* Error code             */

int			NanoSec;	/* Time in ns             */

int			MicroSec;	/* Time in us             */

int			Hz;		/* Frequency              */

int			Mode;		/* Clock mode             */

struct ppsstat	Stat;		/* Pps statistics         */

struct ppsclockev	ClockEvent;	/* Event from pps driver  */

struct ClockData	ClkStatus;	/* Offset data block      */

};

/*

* Complete command structure.

*/

struct ClkCmd {

short			Length;	/* Size of Data + Type    */

short			Type;		/* Command type code      */

union Cmds		Data;		/* Command data block     */

};

The complete message is described by the structure “ClkCmd”, it contains a 16-bit length field followed by a 16-bit type code identifying the message followed by data for the message identified by the type code.

The data part of the message can be empty, contain a integer value or a data structure. All multibyte fields are in network order. ( big endian order ). All command messages sent to microd receive a response message containing a 16-bit status code or response data.

The command messages are :

CLK_OFFSET		Command type 1.
				Update offset and drift values.
				The data field is a “struct ClockData” containing
				the offset and drift values. The leap and status
				fields are not used yet.
		Response :
				A status message containing 0 if all went well
				or a error code otherwise.

CLK_DRIFT			Command type 18.
				Update offset, drift and frequency drift values.
				The data field is a “struct ClockDriftData”
				containing the offset, drift and freq-drift values.
				The leap and status leap and status fields are not
				used yet.
		Response :
				A status message containing 0 if all went well
				or a error code otherwise.

CLK_GETTIMESTAMP	Command type 2.
				Request a PPS timestamp.
				The data field is empty.
		Response :
				A data block containing a “struct ppsclockev”
				or a status message containing a error code.

CLK_GETMICROSTEP	Command type 4.
				Request present microstepper setting in ns.
				The data field is empty.
		Response :
				A data block containing the result as a integer
				or a status message containing a error code.

CLK_GETPPSOUTPUT	Command type 7.

Request present PPS output setting in ns.
				The data field is empty.
		Response :
				A data block containing the result as a integer
				or a status message containing a error code.

CLK_GETMED		Command type 12.
				Request the median value of the last 3 readings
				of the offset between the system time and the
				PPS signal.
				The data field is empty.
		Response :
				A data block containing the result as a integer
				or a status message containing a error code.

CLK_GETHZ			Command type 14.
				Request the present internal clock frequency
				used by the pps interface card.
				The data field is empty.
		Response :
				A data block containing the result as a integer
				or a status message containing a error code.

CLK_GETCLKMODE	Command type 16.
				Request the present clock mode used by the
				pps interface card.
				The data field is empty.
		Response :
				A data block containing the integer 0 when a
				external clock is used and 1 when a internal clock
				is used.
				A status message containing a error code is
				returned if a error occurs.

# Kernel API:s

New functionality has been added to the FreeBSD kernel in the form of a driver receiving the PPS signal from the PPS interface card and a ntp packet filter in the udp protocol. User level programs can communicate with with these functions using various system calls.

## PPS driver API

User level programs communicates with the pps driver with ioctl calls using the /dev/ppsd0 device file.

The interface details are described in the header file ppsclock.h

The following calls are available :

CIOGETEV			Read the last clock event from the driver.
		Response :
				Returns a “struct ppsclockev” containing
				a timestamp and a serial number for the event.
				The timestamp contains the system time when
				the latest pps signal arrived.

CIOGETSTEP			Read present microstepper setting.
		Response :
				Returns a integer containing the microstepper
				setting in ns.

CIOSETSTEP			Update the microstepper.
				The new microstepper setting is suppplied as
				ns in a integer.

Response :
				Status code. 0 if all is well.

CIOGETPPS			Read present pps output setting
		Response :
				Returns a integer containing the pps output
				setting in ns.

CIOSETPPS			Update the pps output.
				The new pps output setting is suppplied as
				ns in a integer.

Response :
				Status code. 0 if all is well.

CIOGETMED			Read the median value of the 3 last offsets
				between the pps signal and system time.
		Response :

A integer offset value in ns.

CIOGETHZ			Read the present clock frequency used on
				the pps interface card.
		Response :

A integer containing the frequency in Hz.

CIOSETHZ			Update the pps card clock frequency.
				The clock frequency is specified in Hz.
				The actual frequency used on the pps card
				is not affetced, only the calculations
				performed by the driver.
		Response :
				A status code. 0 if all is well.

CIOGETCLK			Read the present external/internal status
				of the pps card clock.
		Response :

A integer containing 0 if external clock is used
				or 1 if internal clock is used.

CIOSETCLK			Update external/internal clock status.
				A integer value of 0 sets external mode while
				1 sets internal mode.
		Response :
				A status code. 0 if all is well.

CIOWAITPPS			Wait until next pps pulse.
		Response :
				A status code. 0 if all is well.

The following C-code structs are used :

/*
 *	A PPS clock event used by microd and
 *	the Ntp reference clock.
 */
struct ppsclockev {
	struct timeval	tv;		/* Timestamp */
	u_int			serial;	/* Serial number */
};

/*
 *	PPS driver statistics.
 */
struct ppsstat {
	int	minhw;		/* Min latency value in us*/
	int	maxhw;		/* Max latency value in us*/
	int	minsys;		/* Min clock offset in us */

int	maxsys;		/* Max clock offset in us */

};

## NTP packet filter API

A set of socket ioctl:s are used when communicating with the ntp packet filter. The implemented calls are used to allow/disallow ntp packet types and to read and reset ntp packet statistics maintained by the packet filter.

The interface details are described in the header file checkntp.h

The following ioctl calls are implemented :

SIOCIP_NTPALLOWMODE	Accept ntp packets of the specified type.
					The packet type is specified in a integer.
			Response :
					A status code. 0 if all is well.

SIOCIP_NTPDISALLOWMODE	Reject ntp packet of the specified type.
					The packet type is specified in a integer.
			Response :
					A status code. 0 if all is well.

SIOCIP_NTPCLEARMODES	Reject ntp packets of any type.
			Response :
					A status code. 0 if all is well.

SIOCIP_NTPREADMODES		Read present filter settings.
			Response :
					A integer containg a bitmask with
					0 for rejected and 1 for accepted
					packet types is returned. Bit 0 specifies
					status for packets of type 0, bit 1 for
					type 1 etc.

SIOCIP_NTPREADCOUNTERS	Read present packet filter statistics.
			Response :
					A “struct NtpCnt” is returned containing
					integer counters for packets type 0 to 7.

The following C-code struct is used :

/*
 *	Struct holding 8 integer counters.
 */
struct NtpCnt {
	unsigned int Counter[ 8 ];
};

# Ntp reference clock driver

The reference clock driver interfacing the pps kernel driver to the ntp server is based on the refclock_atom driver. The clock driver is simplified and adapted to use the ioctl interface of the kernel driver to read timestamps.

The clock driver is called refclock_ppsdevice and the C-code is located in the file refclock_ppsdevice.c in the ntpd directory. The driver is integrated using refclock number 32. When specifying the driver in the ntp.conf file the address 127.127.32.0 is used.

The driver opens /dev/ppsd0 when the ntp server is started and the queries the kernel driver once a second to read the timestamps showing the system time at each pps pulse.

The ntp distribution used is 4.0.90f, but ntp distributions 4.0.99c and 3-5.93 with the reference clock driver integrated is also available on the distribution CD.

The ntp distribution is patched to include the clock driver in the “configure” script. When the configuration is performed using “configure –enable-PPSDEVICE” a ntp distribution with the ppsdevice reference clock driver is built.

Typical ntp configuration file ( /etc/ntp.conf ) :

Server 127.127.32.0 prefer

Driftfile /etc/ntp.drift

Keys /etc/ntp.keys

# Utility programs

A number of additional utility programs are available together with the normal ntp distribution. Openssh is installed in order to enable secured remote login. Openssl is also installed to enable SSL connections to the microstepper daemon.

## Fpgaload

A simple utility used when downloading new bitstreams to the Altera Fpga circuit. The download connector on the pps interface card is connected to a PC printerport. A Intelhex bitstream file can then be downloaded using the fpgaload utility.

The complete command syntax for fpgaload is :

fpgaload [-p i/o-port][-r 0/1][-c 0/1][-i][-v][-s] –f bitstream file

-c	Set clock mode, 0 = external, 1 = internal
-f	Specify bitstream file
-i	Read device id and reset polarity
-p	Set printer port base address
-r	Set reset polarity. 0 = active low, 1 = active high
-s	Shift bitstream file
-v	Verify bitstream file against fpga contents

## Setoffset

The setoffset utility can be used to manually send new offset/drift values to the microstepper daemon. The transfer can be performed in clear text or using SSL.

The command syntax is :

setoffset [-ssl] hostname offset drift

The offset is in nanoseconds and the drift value is in picoseconds per day.

## Ntpmode

The ntpmode utililty can be used to manually set which ntp packet types that are to be accepted/rejected. The utility can also be used to check the present packet filter setting and to display the ntp packet filter statistics.

The command syntax is :

ntpmode [-][packet type #]

A negative packet type number rejects the packet type and a positive number accepts the packet type. Using ntpmode without any arguments displays the present setting and the packet filter statistics.

## Logsys

The logsys utility is a primitive tool that displays the present offset and other statistics by querying the pps driver. The interval between queries is specified on the command line.

The command syntax is :

logsys ”interval in seconds”

## Logclk

The logclk utility is a primitive tool that displays the present offset and other statistics by directly querying the hardware. The interval between queries is specified on the command line. Note that the logclk command can interfere with the pps driver since both queries the same hardware registers.

The command syntax is :

logclk ”interval in seconds”

## Setmicro

The setmicro utility updates the microstepper by calling the pps driver. The specified offset is limited to +/- 1 second.

The command syntax is :

setmicro ”offset in nanoseconds”

## Setpps

The setpps utility updates the pps output by calling the pps driver. The specified offset is limited to +/- 1 second.

The command syntax is :

setpps ”offset in nanoseconds”

## Clockmode

The clockmode utility can be used to manually switch the pps interface card clock between external and internal mode. The clock frequency used can also be specified. The specified frequency is used by the pps driver when calculating interrupt latency, microstepper and pps output values.

The command syntax is :

clockmode ext/int [clock frquency]

## Network and ntp performance measurement utilities

These three utilities are designed to be used for measuring ntp performance and network asymmetries. The logd daemon is used to create log files containing readings of the offset between the pps pulse and the system time. The symd/symc pair is used to measure network asymmetries by sending timestamped udp packets from the symc client to the symd server. The symd server adds a timestamp of its own and then returns the packet to the client. The network delay in both directions can then be calculated by the client. The measurement depends on close syncronization of the system clocks of the client and the server.

### Logd

The log daemon creates log files in various formats containing timestamps and the offset between the pps pulse and the system time.

The command syntax is :

logd [-d][-t][-r][-s interval][-f log file]

-d	Use cleartext timestamps in log records
-f	Write log to specified file. Run in daemon mode
-r	Append date to file name and create a new file each day
-s	Time interval in seconds between log records
-t	Use time_t timestamps

### Symd

The symd daemon is the server end of the symd/symc pair. The symd daemon receives timestamps from the symc client, adds a timestamp of its own and returns the packet to the client. If no udp port is specified, port number 4711 is used.

The command syntax is :

symd [udp port #]

### Symc

The symc utility is the client end of the symd/symc pair. The utility sends a timestamped udp packet to the server. When the packet is returned from the server, symc calculates the network delay in both directions.

The command syntax is :

symc –h hostname [-p port][-s interval][-t]

-h	Symd server host name
-p	Port number to use. Default 4711.
-s	Interval in seconds between successive measurements
-t	Include time_t timestamps in log output

# Security strategy

The network security strategy employed consists of the following steps.

- The administration protocol is authenticated using RSA keys and encrypted using 128 bit keys. The protocol uses SSL to enable the authentication and encryption.

- Remote login is also autenthicated using RSA keys and is encrypted using 128 bit keys. Remote login is implemented using ssh. No private keys are stored on the ntp computers, only public keys.

- Ntp requests are filtered in the udp protocol stack. Ntp control messages are not accepted if they arrive on the external ethernet interfaces. If configured, ntp response packets can also be rejected, leaving only ntp queries accepted.

- No other network connectivity is enabled.

The limited network activity leaves very few possibilities open for a hostile attack. The ntp server rejects all control messages and can not be affected by any network activity.

The administration protocol as implemented by the microstepper daemon “microd”, uses SSL with RSA authentication and 128-bit RC4 encryption. It is not possible to connect to the daemon without a valid certificate.

Remote login using ssh also employs RSA authentication. It is not possible to login without the correct private RSA key.

The conclusion is that the ntp server is well protected against attacks from the network.